Hi Darrell,

>

>My apologies on not being able to access the new calculator. We moved it to

>it's new home:

>

><http://carbonchallenge-dev.sr.unh.edu/calculator.jsp>http://carbonchallenge-dev.sr.unh.edu/calculator.jsp

>

>We separated home heating from hot water usage since you mentioned it is

>indeed possible to have an oil heating system with a propane hot water

>system. The reverse isn't likely though, is it? A propane heating system

>with an oil fired hot water system? I wouldn't think that happen but I

>don't know.

>RESPONSE:

>That's right typically propane heating with oil fired hot water is a rarity…..

>

>With all of the action steps we need to quantify the savings in terms of

>percentage of fuel saved. Is it possible to do that? Specifically, I'm

>looking for:

>

>1. Percentage savings in home heating oil for replacing an older (20 years

>or more) oil boiler or furnace with a new Energy Star boiler or furnace.

>The new boiler or furnace could be specified however you want. Ie: new

>Energy Star condensing boiler with an efficiency rating of __%

>RESPONSE:

>This could be a wide range from 4-37 % depending how efficient the old

>unit had been, and whether or not the new unit is an Energy Star unit. A

>fair assumption is to go with the lower end- 15% fuel savings, by going

>with an Energy Star rated oil unit of 86%.

>

>2. Percentage savings in home heating propane/natural gas for replacing an

>older (20 years or more) propane or natural gas boiler or furnace with a

>new Energy Star boiler or furnace. The new boiler or furnace could be

>specified however you want.

>RESPONSE:

>This as well could range from a low of 4% - 37%.

>25% is a conservative figure, if we were to go with  a condensing unit of 94%.

>

>

>3. What would the savings be as a % of fuel use for a flame retension

>burner? Is this applicable to just oil fired systems or does it hold true

>for propane/natural gas systems?

>RESPONSE:

>YES OIL ONLY.

>Flame Retention burner only (recommended only if combustion chamber could

>withstand hotter temperatures of a new burner)...could save 10-20% in

>fuel. 10% savings is a fair rate to use.

>

>

>4. Can sealing ductwork be quantified as a % of fuel use?

>RESPONSE:

>Duct system leaks can lead to 13-25% loss in heated air flow. Attaching a

>10% fuel savings as a result of duct sealing is conservative.

>

>

>5. How about weatherizing by sealing around windows and doors...as a

>percentage of fuel savings?

>RESPONSE:

>This is known as House Doctoring. Data from RS  data provided to American

Council for an Energy Efficient Economy (ACEEE) cites an Energy savings of

>10%.

>

>

>

>6. Lastly, how about insulating a hot water tank with an insulating

>blanket... what percentage of fuel use would that save?

>RESPONSE:

>If insulate both hot water tank and hot water pipes --annual savings 5%

>could be realized.

>

>With all these, Darrell, I know it's difficult to pinpoint exact savings

>because it depends on a number of factors. I'm just trying to get a

>reasonable estimate (perhaps on the conservative side) of what someone can

>expect in fuel savings for taking these actions.

>

>Thanks so much for your help on this!

>

>Denise